# MiWay Route Efficiency Analysis

Professional summary of GTFS-based route metrics and advocacy priorities

| Prepared for | UTM Transit Housing Advocacy |
| --- | --- |
| Dataset | MiWay GTFS scheduled service feed, valid May 1, 2026 to June 28, 2026 |
| Representative day | Tuesday, May 5, 2026 |
| Analysis scope | Scheduled service, route geometry, speed, frequency, and route-level scoring |

Bottom line This analysis converts MiWay's scheduled GTFS feed into a transparent route efficiency ranking and an advocacy priority list. The strongest routes combine high scheduled speed, frequent service, long service span, and balanced bidirectional service. The priority list highlights routes where improvement could matter because the route has meaningful service volume but weaker efficiency indicators.

| Routes analyzed | Weekday trips | All-day routes | Score range |
| --- | --- | --- | --- |
| 66 | 5,507 | 52 | 17.46-84.14 |

# Executive Summary

The project analyzed MiWay scheduled service using a GTFS feed valid from May 1, 2026 to June 28, 2026. To avoid mixing weekday and weekend patterns, the analysis uses Tuesday, May 5, 2026 as the representative service day.

Across 66 active routes, the final route efficiency score ranges from 17.46 to 84.14, with a median score of 52.27. The system-level average scheduled route speed is 22.12 km/h, and the median route-level scheduled headway is 23.75 minutes.

The highest-scoring routes are not simply the routes with the most trips. They tend to combine speed, all-day coverage, relatively frequent service, and balanced directions.

The advocacy priority lens surfaces routes that may deserve operational attention because they have substantial service or rider relevance but weaker efficiency indicators.

This ranking should be treated as a screening tool. It does not replace ridership, on-time performance, crowding, equity, or rider experience data.

# What the Five Notebooks Produced

| Notebook | Focus | What it added |
| --- | --- | --- |
| 1 | GTFS exploration | Loaded source tables, confirmed feed dates, selected Tuesday, May 5, 2026, and isolated active weekday trips. |
| 2 | Route geometry metrics | Calculated route length, directness, unique stops, average stops per trip, and stops per kilometre. |
| 3 | Scheduled speed and travel time | Parsed GTFS times beyond midnight, calculated trip duration, distance, and scheduled speed by route. |
| 4 | Frequency and availability | Calculated first and last departures, service span, headways, period-level trips, all-day service, and directional balance. |
| 5 | Final efficiency ranking | Combined geometry, speed, and service metrics into final scores, ranks, score bands, and advocacy priority scores. |

# Scoring Method

The final score uses percentile-based component scores, which means each route is compared against other MiWay routes in the same feed. Higher scores represent stronger scheduled performance relative to the rest of the system on the selected weekday.

| Component | Weight | Interpretation |
| --- | --- | --- |
| Scheduled speed | 25% | Higher average scheduled speed scores better. |
| Frequency | 25% | Lower median headway and higher trip volume score better. |
| Service span | 15% | Longer daily span from first departure to last arrival scores better. |
| Route directness | 15% | More direct non-loop route geometry scores better. |
| Stop-spacing efficiency | 10% | Fewer stops per kilometre scores better as an operating efficiency signal. |
| Directional balance | 10% | More even service between directions scores better. |

Important caveat Stop density is an operating efficiency signal, not a service quality verdict. More stops can slow a route, but they can also improve access for riders. Similarly, loop routes can appear geometrically inefficient because their endpoints are close together; the notebook neutralizes directness for near-loop routes to avoid penalizing that design artifact.

# Final Route Efficiency Ranking

The table below shows the ten highest-scoring routes in the final ranking. These routes generally combine strong speed, frequent service, long span, and balanced directions.

| Rank | Route | Name | Score | Trips | Speed | Headway |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 11 | Westwood | 84.14 | 172 | 35.64 | 15.00 |
| 2 | 35 | Eglinton | 77.54 | 159 | 24.61 | 15.00 |
| 3 | 1 | Dundas | 74.65 | 172 | 21.84 | 15.00 |
| 4 | 109 | Meadowvale Express | 72.72 | 162 | 35.02 | 15.00 |
| 5 | 42 | Derry | 72.30 | 171 | 22.33 | 15.00 |
| 6 | 66 | McLaughlin | 71.17 | 202 | 20.97 | 12.00 |
| 7 | 110 | University Express | 71.15 | 118 | 31.64 | 21.00 |
| 8 | 61 | Mavis | 69.75 | 189 | 21.32 | 14.00 |
| 9 | 107 | Malton Express | 69.57 | 147 | 26.25 | 15.00 |
| 10 | 101 | Dundas Express | 68.90 | 137 | 21.46 | 15.00 |

The score bands show that the system has a wide spread of scheduled performance. In this scoring version, 7 routes fall in the high band, 22 in moderate, 17 in low, and 20 in very low.

# Routes for Advocacy Review

The advocacy priority score is intentionally different from the efficiency rank. It prioritizes routes with weaker efficiency scores and meaningful service volume, because improvements on those routes may affect more riders.

| Priority | Route | Name | Priority score | Efficiency | Trips |
| --- | --- | --- | --- | --- | --- |
| 1 | 28 | Confederation | 68.44 | 37.96 | 129 |
| 2 | 6 | Credit Woodlands | 65.01 | 29.03 | 93 |
| 3 | 10 | Bristol | 64.36 | 36.16 | 102 |
| 4 | 2 | Hurontario | 64.02 | 59.96 | 232 |
| 5 | 8 | Cawthra | 62.99 | 26.33 | 71 |
| 6 | 3 | Bloor | 60.81 | 62.28 | 180 |
| 7 | 4 | North Service Road | 60.37 | 24.64 | 55 |
| 8 | 31 | Ogden | 57.42 | 45.71 | 100 |
| 9 | 30 | Rexdale | 57.28 | 23.73 | 48 |
| 10 | 61 | Mavis | 56.94 | 69.75 | 189 |

The highest-priority review routes are not automatically the worst routes. They are routes where the schedule suggests an advocacy conversation could be productive: speed may be low, headways may be long, stop density may be high, or directness may be weak while the route still carries substantial scheduled service.

# Key Findings

Service availability is broad but uneven. The feed contains 5,507 scheduled weekday trips across 66 active routes, and 52 routes meet the all-day service flag.

Scheduled speed varies sharply. Route-level average scheduled speed ranges from 14.95 km/h to 38.17 km/h.

Frequency is not evenly distributed. The median route-level headway is 23.75 minutes, but some routes have median headways above 40 minutes.

Several important corridors show up in the advocacy priority lens, including Confederation, Credit Woodlands, Bristol, Hurontario, Cawthra, Bloor, and North Service Road.

School routes and special-purpose routes require separate interpretation because they are not designed to behave like all-day grid routes.

# Advocacy Implications

For transit and housing advocacy, the most useful conclusion is not only which route ranks first. The stronger question is where better service would unlock access to housing, campus, jobs, groceries, and late-day travel.

Use the priority list to identify candidate corridors for deeper public-facing analysis, especially routes with high trip volume and weak efficiency indicators.

Ask MiWay and the City of Mississauga for ridership, crowding, on-time performance, pass-up, and stop-level boarding data so scheduled efficiency can be compared with lived rider experience.

Pair route efficiency results with housing density, student housing locations, major employment areas, and late-night travel needs.

Evaluate whether priority routes need bus priority, stop consolidation, branch simplification, schedule changes, or better transfer coordination.

Separate school-special and limited-purpose routes from all-day local routes when communicating results, because their service design goals differ.

# Limitations

The analysis uses scheduled GTFS data, not real-time vehicle movement or actual traffic conditions.

The ranking does not include ridership, crowding, fare equity, accessibility barriers, safety, or on-time reliability.

A single representative weekday keeps the project clean, but weekend and seasonal patterns should be studied separately.

The final score is transparent but still depends on chosen weights. The weights should be revisited with stakeholders before using the score for formal policy recommendations.

Lower stop density can improve speed, but stop removal can harm riders with mobility constraints or long walks. Any stop-spacing recommendation needs an equity review.

# Recommended Next Steps

Create a map-based appendix showing the top advocacy-priority routes and nearby housing or campus destinations.

Build a sensitivity test that compares alternate scoring weights, such as a rider-access score versus an operating-efficiency score.

Add weekend notebooks for Saturday and Sunday to separate commuter, student, retail, and shift-worker travel patterns.

Compare scheduled speed with real-time travel time data if vehicle location or historical performance feeds become available.

Turn the final route table into a short public brief with 3 to 5 corridor-specific asks.

# Appendix: Lower-Scoring Routes

The routes below have the lowest final scores in this first scoring version. These results should be interpreted with service purpose in mind. A school route, loop route, or coverage route may score low while still serving a valid equity or access function.

| Rank | Route | Name | Score | Trips | Speed | Headway |
| --- | --- | --- | --- | --- | --- | --- |
| 66 | 321 | Stephen Lewis SS | 17.46 | 5 | 14.96 | 33.00 |
| 65 | 30 | Rexdale | 23.73 | 48 | 19.14 | 55.00 |
| 64 | 4 | North Service Road | 24.64 | 55 | 19.86 | 44.00 |
| 63 | 313 | Streetsville SS | 24.68 | 3 | 20.39 | n/a |
| 62 | 8 | Cawthra | 26.33 | 71 | 18.60 | 35.00 |
| 61 | 16 | Malton | 27.34 | 50 | 19.34 | 40.00 |
| 60 | 6 | Credit Woodlands | 29.03 | 93 | 18.39 | 28.00 |
| 59 | 315 | Rick Hansen SS | 32.05 | 3 | 14.95 | 4.00 |
| 58 | 46 | Tenth Line | 32.31 | 62 | 19.73 | 44.00 |
| 57 | 304 | Father Goetz SS | 34.63 | 1 | 17.43 | n/a |

# Appendix: Files Produced

Notebooks/01_miway_gtfs_exploration.ipynb

Notebooks/02_miway_route_geometry_metrics.ipynb

Notebooks/03_miway_scheduled_speed_travel_time.ipynb

Notebooks/04_miway_frequency_service_availability.ipynb

Notebooks/05_miway_final_route_efficiency_ranking.ipynb

Outputs/final_route_efficiency_ranking_2026-05-05.csv

Outputs/advocacy_priority_routes_2026-05-05.csv

Outputs/route_efficiency_score_components_2026-05-05.csv